Upgrade motherboard through Jlink

ST_STM32F207ZG:

- Prepare the Jlink tool and locate the burn port(usually close to the chip).

- Connect the tool to the port.

- Open “J-Flash V5.02k” and select “Create a new project”, then click “Start J-Flash” button.

- Configure project settings.

- Click “Options”-->”Project settings”.

- In “Project settings”, click “Target Interface”, select “SWD” in the second red box, then select “Auto selection” in the third and fourth red boxes.

- In “Project settings”, click “CPU”, select “Device” and click the third red box to open “Select device” page. In “Select device” page, select device “STM32F207ZG” then click ok.

- Select upgrade file. Click “File” -> “Open data file” -> select the corresponding hex file -> “Open”.

- Start upgrade. Click Target -> Auto to start upgrade. After upgrade, click “Start application” to start the charging pile(Note: If the upgrade is successful, you will hear the motherboard emit a beep sound).

Renesas_R7FA6M4AF:

- Prepare the Jlink tool and locate the burn port(usually close to the chip).

- Connect the tool to the port.

- Open “JFlash-Lite V6.98c”, click the first red box, select “R7FA6M4AF”, then click ok.

- Click the first red box, select hex file in “Open data file” page, then click open.

- Click “Program Device” to start upgrade(Note: If the upgrade is successful, you will hear the motherboard emit a beep sound).

- If the internal program of the charging pile is damaged and unable to start due to flashing the wrong firmware or other reasons, please follow the following steps to operate.

- Erase the program inside the chip. Click “Target” -> “Erase chip”.

- Upgrade Boot file(The file type is hex). Open Bootloader file, follow step 7 to start upgrade Bootloader file.

- Upgrade firmware. Click “File” -> “Open data file” -> select the corresponding hex file -> “Open”. Then follow step 7 to start upgrade firmware.

- After upgrade, open Parameters of Charging Pile Web Page, select Machine Type and Power Meter Type.

Written by  Peaken

Checked by Truman

Reviewed by    Tom